| 履历页 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 序号 | 更改内容摘要描述 |  | 版本 | 修订人 | 修订日期 |
| 1 | First release |  | 1.0 | Jason | 2025.05.26 |
| 批 准： 日 期： |  | 审 核： 日 期： | 制 作： 日 期：2024-04-03 |  |  |

1. Install APP

- 1.1 Import the installation file "avoip_1.0.xxxx.apk" into the PAD.

- 1.2 Find the installation file above in "My files", double-click the file name to start installing the app.

- 1.3 After the app installation is completed, the following icon can be seen in the system.

2. Setup testing environment

- The system installation topology diagram is as follows:

- Video LAN port of the controller, encoder, and decoder are all connected to the switch.

- Connect the Control LAN port of the controller to the router.

- Connect the PC to the router and set the network card to DHCP mode. PC can access the controller through Web GUI.

- Android phone/pad is connected to the router via WIFI. Android phone/pad can access the controller through the app.

3. Access the controller through app

- 3.1 Open the app, and the main interface will display as follows:

- the search function on the left and the device list on the right.

- 3.2 Click the Start Scanning button to start searching for devices, wait for the devices to be found (as shown in the figure below), and then click the button again to stop the search.

- 3.3 Press and hold the device icon in the device list, and the button Edit and Delete will pop up.

- Edit can modify the name of the device.

- Delete can remove a device from the device list.

- 3.4 Click on the device icon in the device list to access the control box, as shown below:

- Enter your account and password to login the main interface. Other operations are the same as in the Web GUI.

- Click the button "<" in the above picture to return to the device list.

- In the login interface, if Remember me is checked, the current login account and password will be remembered. The next time you access the control box from the device list, you will skip the login page and directly enter the main interface. Exiting login on the main interface will clear the remembered account and password. The logout button is shown in the following figure.

- 3.5 If the device cannot be found, you can also click the button Manually Add to manually add the device.

- Enter the name, IP, and port number 80 of the controller.

- After clicking OK, a new device was added, as shown in the following figure:

- The method for manually adding device access control boxes is the same as before.