# User Manual

CVHU3

HDMI to USB Video Capture Device

All Rights Reserved

Version: CVHU3_2020V1.1

HDMI to USB Video Capture Device

Introduction

The CVHU3 is a HDMI to USB video capture device. The product is designed to capture video and audio signal from the HDMI source. The content captured can be saved to the PC/Laptop through a USB cable.

# Features

- Capture video and audio from HDMI sources.

- HDMI video resolution is up to 4K@60Hz 4:4:4, HDR10

- Supports HDMI loop output for display device connection.

- Capture resolutions up to 1080p at 60 frames per second through USB port.

- Compatible with multiple third-party applications, such as OBS/ VLC/Potplayer/ YouTube on Windows, QuickTime on Mac OS X.

- Supports microphone input and headphone output.

- Powered from USB port, no external power supply required.

# Package List

Please make sure all the items listed below are in the package. If anything is missing or damaged, please contact your distributor for a replacement.

- 1x CVHU3 HDMI to USB Video Capture Device

- 1x USB Cable

- 1x User Manual

# Panel Description

1

2

3

4	5

- HDMI In: Connects to HDMI source device.

- HDMI Out: HDMI loop output for HDMI display device connection.

- USB Record: Connects to laptop, PC or other recording device.

- Earphone: Connects to headphone for audio de-embedding.

- MIC: Connects to microphone for audio mixing.

HDMI to USB Video Capture Device

System Connection

## System Requirement

  - The recording device with available type-A USB port.

  - Microsoft Windows, Mac OS X or Linux.

  - HDMI audio/video source with HDMI Cable (Camera, Xbox, PS4, DVD…etc.).

## System Diagram

HDMI to USB Video Capture Device

Recording for Windows using VLC

VLC Media Player Initial Setting

Note: The VLC Media Player is the recommended software used for video capturing. Please visit

www.videolan.org/vlc/ to download VLC Media Player for Windows, Mac OS X or Linux.

For initial use, the hardware accelerated decoding must be set at “Automatic” to ensure the best performance. Please following the instructions below to set the hardware accelerated decoding.

- Select “Tools”, and then click on “Preferences” at the drop down menu.

- Click “Input/Codes”, and the select “Automatic” for Hardware-accelerated decoding, and then “Save” to finish the initial VLC setting.

HDMI to USB Video Capture Device

VLC Media Player Direct Show

- Select “Media”, and then select “Open Capture Device” at the drop down menu.

- For video device name, select “USB3.0 HD Video Capture”.

- For audio device name, select “Microphone (USB3.0 HD Audio Capture)”.

- To set up the correct aspect ratio, click “Advanced options”.

- Enter the aspect ratio for the video source (e.g. 16:9).

- Click “OK” to exit the “Advanced options” menu.

- Click “Play”, the source screen will show in approximately several seconds.

HDMI to USB Video Capture Device

VLC Media Player Recording

Note: We recommend using the“Convert/Save” method to record the video from the device for best performance.

- Select “Media”, and then select “Convert / Save” at the drop down menu.

- Click “Capture Device” tab on the top right side.

- Video device name should be “USB3.0 HD Video Capture” and audio device name should be “Microphone (USB3.0 HD Audio Capture)”.

- To set the aspect ratio, click “Advanced options”.

- Enter the aspect ratio for the video source (e.g. 16:9).

- Click “OK” to exit the “Advanced options” menu.

- Click “Convert /Save”.

- Select “Display the output” check-box, and then click “Browse” to choose where to save the destination file and enter the file name.

- Click “Start”, and the video and audio recording will start in approximately several seconds.

- To stop recording, simply click the square “Stop” button at the bottom control panel.

HDMI to USB Video Capture Device

Recording for Mac using QuickTime

- Launch QuickTime Player, click “File” at the top menu and select “New Movie Recording”.

- The recording control panel will appear on the screen, first click the “∨” symbol for the drop down menu.

- Select “USB3.0 HD Video Capture” under “Camera”, and select “USB3.0 HD Audio Capture” under “Microphone”. Then adjust to desired volume for the speaker.

- Click the “Record” button to start recording.

- To stop recording, simply click the square “Stop” button.

- Enter the recording file name and select where to save the file.

HDMI to USB Video Capture Device

Specification

| Model | CVHU3 |  |
| --- | --- | --- |
| Description | HDMI to USB Video Capture Device |  |
| Input | (1) HDMI In, (1) Microphone |  |
| Input Connector | (1) Type-A HDMI, (1) 3.5mm jack |  |
| Output | (1) HDMI Out, (1) USB Record, (1) Earphone |  |
| Output Connector | (1) Type-A HDMI, (1) Type-A USB, (1) 3.5mm jack |  |
| HDMI Standard | 2.0 |  |
| HDMI Input Resolution | Up to 4K@60Hz 4:4:4, HDR10 |  |
| HDMI Output Resolution | Up to 4K@60Hz 4:4:4, HDR10 |  |
| USB Output Color Space | YUV2, RGB |  |
| HDMI Audio Format | LPCM 7.1 audio, Dolby Atmos®, Dolby® TrueHD, Dolby Digital® Plus, DTS:X™, and DTS-HD® Master Audio™ pass-through. |  |
| Microphone Audio Format | PCM 2.0 |  |
| Headphone Audio Format | PCM 2.0 |  |
| OS Support | Windows, Mac OS X, Linux. |  |
| Compatible Media Player | OBS, Potplayer, VLC and so on. |  |
| Video Capture Format | HDMI Source | Captured Video by USB |
|  | 4K@60Hz 4:4:4 | 1080p@60Hz/720p |
|  | 1080p@60Hz 4:4:4 | 1080p@60Hz/720p |
|  | 720p | 720p |
|  | 480 | 480 |
|  | 576 | 576 |
| General |  |  |
| Operation Temperature | -5 to +55℃ (+23° to +131°F) |  |
| Storage Temperature | -25 to +70℃ (-13° to +158°F) |  |
| Relative Humidity | 10% to 90%, Non-condensing |  |
| Dimension (W*H*D) | 102mm x 13mm x 61mm |  |
| Net Weight | 75g |  |

Altimium By Tesca

587, avenue Blaise Pascal 01.64.13.31.00