# ALT-D2AUE

VER 1.0

Thank you for purchasing this product

For optimum performance and safety, please read these instructions carefully before connecting, operating or adjusting this product. Please keep this manual for future reference.

Surge protection device recommended This product contains sensitive electrical components that may be damaged by electrical spikes, surges, electric shock, lighting strikes, etc. Use of surge protection systems is highly recommended in order to protect and extend the life of your equipment.

Table of Contents

- Introduction.	1

- Features.	1

- Package Contents.	1

- Specifications.	2

- Operation Controls and Functions.	3

- Application Example.	4

# Introduction

This Dante 2CH Analog Audio Encoder is an audio transmitter based on the Dante Ultimo solution. It supports balanced/unbalanced audio input and 3- level gain adjustment. It encodes audio signals, and then transmits the audio to the receiving end through network (Dante). It supports POE function.

# Features

☆ Analog audio input supports balanced/unbalanced audio input (unbalanced access to "+" and "GND")

☆ Audio sampling rate supports 44.1kHz, 48kHz and 96kHz (24bit)

☆ Dante solution supports AES67 RTP audio transmission

☆ Audio input supports 3-level gain adjustment：

+18dBu, 0dBu (default) and -10dBV

☆ Support DC 12V power supply and POE function (IEEE802.3af Class 0)

# Package Contents

① 1× Dante 2CH Analog Audio Encoder

② 1× 6pin-3.81mm Phoenix Connector (male)

③ 1× 2pin-3.81mm Phoenix Connector (male)

④ 1× User Manual

# Specifications

| Technical |  |
| --- | --- |
| Video Network Bandwidth | 100M |
| Audio Formats | PCM 2.0 44.1kHz/48kHz/96kHz up to 24bit |
| Transmission Distance | 100m (CAT6/6A) |
| Control Method | Dante Controller |
| Connection |  |
| Inputs | 1x PWR [2-pin phoenix, 3.81mm] 1x AUD [6-pin phoenix, 3.81mm] |
| Output | 1x DANTE [RJ45 connector, POE] |
| Mechanical |  |
| Housing | Metal Enclosure |
| Color | Black |
| Dimensions | 120mm[W] x 47mm[D] x 26mm[H] |
| Weight | 186g |
| Power Supply | DC input: 12V POE input: IEEE802.3af Class 0 |
| Power Consumption | 1.2W |
| Operation Temperature | 32 - 104°F / 0 - 40°C |
| Storage temperature | -4 - 140°F / -20 - 60°C |
| Relative Humidity | 20 - 90% RH (no condensation) |

# Operation Controls and Functions

1

2

3

4

| No. | Name | Function Description |
| --- | --- | --- |
| 1 | DC IN | DC 12V power input port. |
| 2 | AUDIO INPUT (CH1/CH2) | 2 channel analog audio input ports, supporting balanced and unbalanced audio. |
| 3 | DANTE (POE) | Dante digital audio output port. Connect to the Network Switch through RJ45 line, supporting PD power supply. The green LINK LED is always on after normal connection. The yellow DATA LED is flashing when there is data transmission. |
| 4 | GAIN switch | Used for audio input gain adjustment, respectively +18dBu, 0dBu (default), -10dBV. |

# Application Example

USB Audio Encoder/Decoder

US

Dante Audio

B Audio

Laptop

100/1000M Network Switch

Dante

Audio  Audio Encoder

L/R Audio

DVD

Audio Decoder

udio

Dante A

L/R Audio

Speakers

Trademarks

Dante® is registered trademark of Audinate Pty Ltd. All other trademarks are the property of their respective owners.