ALT-SHARE

Quick Start Guide v1.1

# front panel:

rear panel:

1

2

Reserved for future use Mute button

3

4

5

6

Switch the source from USB 2.0 Switch the source from USB 3.0 Switch the source from HDMI Single view layout

7

8

9

10

Side by side layout Big & small layout 1x USB 2.0 camera 1x USB 3.0 camera

11

12

1x HDMI camera or video source Reset

13

1x HDMI 2.0 output for screen

14

15

1x USB-C output for PC 1x line in

16

1x line out

17

1x RS232 for control

18

1x 1000M LAN for control and PoE

# System Diagram:

Specifications:

-Supports 3 camera sources connection, 1x USB 3.0, 1x USB 2.0, 1x HDMI 1.4

-HDMI input can be any HDMI source like laptop, document camera

-Seamless switching between 3 input sources

-Mix videos from 3 input sources

-Mixed layout can be defined in the web like picture in picture, side by side, big and small.

-1x HDMI 2.0 output for local monitor, 1x USB-C (UVC/UAC) output for PC

-Enable mute video/audio to video conferencing or live broadcasting

-Support overlay company logo or text on the output video (future firmware)

-Audio I/O and mixer through the web

-Controllable by LAN & RS-232

-PoE supported, saving additional PSU

-Cost-effective, reliable and simple to install (no driver required)

-Compact and slim housing design with only 18mm height

# Front user button:

Quick to Start

## Power on & Video Switching.

  - Run video conferencing software, such as Zoom, Microsoft Teams, etc.

Note: the HDMI output will show the default homepage without camera activated, it will sync the video image once camera source is selected during video conferencing.

  - Select [VM10 Camera], will output the video source from USB 2.0 by default

  - Click the [button 2], switch the source from USB 3.0

  - Click the [button 3], switch the source from HDMI

## Mixed layout

  - Single view layout

  - Side by side layout.

-firstly click this button to enter this layout, then click any 2 buttons of 3 video sources to make them mixed

-double click the button to swap the position of two sources

  - Big & small layout, double click the button to swap the position of two sources

-firstly click this button to enter this layout, then click any 2 buttons of 3 video sources to make them mixed

-double click the button to swap the position of two sources

  - Enable mute video/audio to video conferencing or live broadcasting

## Audio I/O and setting through the web

  - use the LAN IP address indicated in the home page to enter the Web UI.（Password is admin）

  - Click the [Audio Setting]

  - Select the audio from corresponding sources as UAC signal, supporting volume control and mute for each source.

-part 1 is for testing speaker and microphone from connected sources

-part 2 is for selecting source for speaker out

-part 3 is for selecting sources for mic in, can be mixed with difference sources.

  - The [Success] prompt box will pop up on the right when the setting is completed.