# TP100WP2HU-UK

Wall-plate Transmitter

HDBT Receiver

VER 1.0

Thank you for purchasing this product

For optimum performance and safety, please read these instructions carefully before connecting, operating or adjusting this product. Please keep this manual for future reference.

Surge protection device recommended

This product contains sensitive electrical components that may be damaged by electrical spikes, surges, electric shook, lighting strikes, etc. Use of surge protection systems is highly recommended in order to protect and extend the life of your equipment.

Table of Contents

- Introduction.	1

- Features.	1

- Package Contents.	2

- Specifications.	2

- Operation Controls and Functions.	3

  - Transmitter Panel.	3

  - Receiver Panel.	5

  - Dimensions of Wall Box.	6

- Application Example.	8

# Introduction

The 4K60 HDMI/USB-C 2x1 HDBT Wall-plate Transmitter supports one HDMI input and one USB-C input within a standard 86mm×86mm wall box. It can transmit HDMI high-definition signal, bidirectional IR control signal, bidirectional RS-232 control signal and USB 2.0 signal over CAT6 to the distance up to 328ft/100m (HBT mode) or 492ft/150m (LRM mode). Video resolution is up to 4K2K@60Hz (4:4:4).

This product converts HDMI signal to standard HDBaseT signal and transmits it through LAN cable. It can easily control signal source device or display device from the remote end through bi-directional IR signal pass- through function. It also supports POC function with POC control switch.

The Extender can be widely used in other fields such as video conference system, multimedia signal broadcasting, HDMI signal extension, etc.

# Features

☆ HDMI 2.0b, HDCP 2.2/1.x and DP 1.2a compliant

☆ HDMI input supports 18Gbps video bandwidth USB input supports 5.4Gbps video bandwidth

☆ Support video resolution up to 4K2K@60Hz (4:4:4)

☆ Up to 7.1 channels HD audio

☆ Input signal source selection supports button switching

☆ Built-in mode DIP switch for selecting the HBT mode or LRM mode

HBT mode: Audio and video signals can be transmitted up to 328ft/100m for 4K60/4K30/1080P

LRM mode: Audio and video signals can be transmitted up to 492ft/150m for 4K30/1080P

☆ Built-in EDID DIP switch supports intelligent EDID management

☆ Built-in audio DIP switch supports analog audio embedding

☆ Built-in POC switches, when both POC switches of TX and RX are turned on, the Extender supports bidirectional 24V POC function, that is, either TX or RX is powered by 24V1A power supply, the another does not need power supply from the DC jack. (See the Description 1)

☆ Transmit bidirectional Infrared control signal together with the HDMI signal. (See the Description 2)

IR IN is a 5V power supply level

☆ Transmit bidirectional RS-232 control signal together with the HDMI signal. (See the Description 3)

☆ The Receiver supports USB 2.0 signal transmission

☆ Standard 86mm×86mm wall box

# Package Contents

① 1 x 4K60 HDMI/USB-C 2x1 HDBT Wall-plate Transmitter

② 1 x HDBT Receiver

③ 2 x 2pin-3.81mm Phoenix connector (male)

④ 4 x 3pin-3.81mm Phoenix connector (male)

⑤ 1 x Wideband IR Receiver cable (1.5 meters)

⑥ 1 x IR Blaster cable (1.5 meters)

⑦ 2 x Mounting Ears

⑧ 4 x Machine Screws-M3*4

⑨ 1 x 24V/1A Locking Power Supply

⑩ 1 x User Manual

# Specifications

| Technical |  |
| --- | --- |
| HDMI Compliance | HDMI 2.0b |
| HDCP Compliance | HDCP 2.2 |
| Video Bandwidth | HDMI: 18Gbps; USB-C：5.4Gbps |
| HDMI Video Resolution | Up to 4K2K@60Hz 4:4:4 |
| Color Space | RGB 4:4:4, YCbCr 4:4:4, YCbCr 4:2:2, YCbCr 4:2:0 |
| Color Depth | 8/10/12bit |
| HDMI Audio Formats | LPCM 7.1CH, Dolby True HD and DTS-HD Master (Full format) |
| Analog Audio Embedding | 48KHz |
| Transmission Distance | LRM mode: 492ft/150m (4K30/1080P) STD-HBT mode: 328ft/100m (4K60/4K30/1080P) |
| ESD Protection | Human body model—±8kV (Air-gap discharge) &±4kV (Contact discharge) |
| Connections |  |
| Transmitter | Input: 1 x HDMI IN [Type A, 19-pin female] 1 x USB-C IN [USB Type C] 1 x AUDIO IN [3.5mm Stereo Mini-jack] Output: 1 x HDBaseT OUT [RJ45] Control: 1 x TO PC [USB Type B] 1 x RS-232 [3pin-3.81mm Phoenix connector] 1 x IR IN [3pin-3.81mm Phoenix connector] 1 x IR OUT [2pin-3.81mm Phoenix connector] |
| Receiver | Input: 1 x HDBT IN [RJ45] Output: 1 x HDMI OUT [Type A, 19-pin female] 1 x AUDIO OUT [3pin-3.81mm Phoenix connector] Control: 2 x USB 2.0 [USB Type A] 1 x RS-232 [3pin-3.81mm Phoenix connector] 1 x IR IN [3pin-3.81mm Phoenix connector] 1 x IR OUT [2pin-3.81mm Phoenix connector] 1 x SERVICE [Micro USB] |

| Mechanical |  |
| --- | --- |
| Housing | Metal Enclosure |
| Color | Front panel: White; Rear case:Black |
| Dimensions | Transmitter: 86mm [W] x 86mm [D] x 38.5mm [H] Receiver: 157.2mm [W] x 88mm [D] x 20mm [H] |
| Weight | Transmitter: 234g; Receiver: 377g |
| Power Supply | 24V/1A (supporting bidirectional POC power supply) |
| Power Consumption | 12.48W (Max) |
| Operating Temperature | 32 - 104°F / 0 - 40°C |
| Storage Temperature | -4 - 140°F / -20 - 60°C |
| Relative Humidity | 20 - 90% RH (no condensation) |

# Operation Controls and Functions

USB-C IN

1

USB-C

HDMI

2

4

3

13

HDMI IN

OFF

HDBaseT OUT

RS-232  IR IN  IR OUT DC 24V RX TX  5V S

12

ON

AUDIO IN  TO PC  SOURCE

5

6

7

8

9  10 11

LRM

STD

HDMI

0

1

MODE   AUDIO IN	1 2

EDID

14

15

16

POC

Transmitter panel

| No. | Name | Function Description |
| --- | --- | --- |
| 1 | USB-C IN port | USB-C signal input port, supporting video signal (up to 4K60) and USB2.0 signal, connected to USB-C signal source device. |
| 2 | USB-C LED | USB-C signal input indicator. When the USB-C signal is selected as the input source, the green indicator light on. |
| 3 | HDMI IN port | HDMI signal input port, connected to HDMI source device, such as Blu-ray or PS4. |
| 4 | HDMI LED | HDMI signal input indicator. When the HDMI signal is selected as the input source, the green indicator light on. |
| 5 | AUDIO IN port | Analog audio input port for audio embedding. |
| 6 | TO PC port | USB-B port, connected to PC. This port can be switched along with the HDMI IN port, that is, when switching to the HDMI signal source, the USB HOST can be selected simultaneously. |
| 7 | SOURCE button | Used to switch HDMI/USB-C input signal source. |
| 8 | RS-232 port | Connected to PC or control system through 3-pin Phoenix connector for RS-232 serial command pass-through. |
| 9 | IR IN port | Connected to IR wideband receiver cable, receiving the remote control signal and then send it to the receiving end. |
| 10 | IR OUT port | Connected to IR blaster cable, sending the remote control signal from the receiving end. |
| 11 | DC 24V port | DC 24V/1A power input port. Note that the extender supports POC function, it means that either transmitter or receiver is powered on by 24V/1A power adapter, the other one doesn’t need power supply. |
| 12 | POC switch | Use the switch to turn on/off POC function. |
| 13 | HDBaseT OUT port | HDBaseT signal output port, connected to the HDBT IN port of Receiver with a CAT6 cable. |
| 14 | MODE DIP switch | Used to select the working mode: LRM-long reach mode: Extend the signal up to 492ft/150m through a single CAT6 network cable. STD-HBT mode: Extend the signal up to 328ft/100m through a single CAT6 network cable. |
| 15 | AUDIO IN DIP switch | Used to select the signal source for the audio output of Receiver: HDMI: Output the de-embedded audio from the HDMI IN port of Transmitter. L/R: Output the analog audio embedded from the AUDIO IN port of Transmitter. |

| No. | Name | Function Description |
| --- | --- | --- |
|  |  | Used to set the EDID information: |
| 16 | EDID DIP switch | 00- Copy RX display’s EDID (as factory default) 01- 4K30 4:4:4 10- 1080p60 4:4:4 |
|  |  | 11- 1200p60 4:4:4 |

1

2

SERVICE

LINK  VIDEO

USB 2.0

USB 2.0

1	2	3

4

5

ON  OFF

L	R

RX

DC 24V	POC

HDMI OUT  AUDIO OUT  RS-232	IR IN  IR OUT

6

7

9  8 10

11

12

13

14	15

HDBT IN

Receiver panel

| No. | Name | Function Description |
| --- | --- | --- |
| 1 | Power LED | Red LED indicates that the Receiver is powered on. |
| 2 | LINK LED (Green) | - Light on: Transmitter and Receiver are in good connection status. - Light Flashing: Transmitter and Receiver are in poor connection status. - Light off: Transmitter and Receiver are not connected. |
| 3 | VIDEO LED (Green) | Signal indicator - Light Flashing: There is valid signal input. - Light off: There is no signal or the signal is invalid. |
| 4 | USB 2.0 port | USB Type A port for connecting USB 2.0 devices, such as USB Flash Drive, Mouse and Keyboard. |
| 5 | SERVICE port | Software update port. Send command “s switch uart 0!” to switch MCU; send command “s switch uart 1!”to switch VALENS. |
| 6 | DC 24V port | DC 24V/1A power input port. Note that the extender supports POC function, it means that either transmitter or receiver is powered on by 24V/1A power adapter, the other one doesn’t need power supply. |

| No. | Name | Function Description |
| --- | --- | --- |
| 7 | POC switch | Use the switch to turn on/off POC function. |
| 8 | HDBT IN port | HDBaseT signal input port, connected to the HDBaseT OUT port of Transmitter with a CAT6 cable. |
| 9 | Link Signal Indicator (Green) | - Light on: Transmitter and Receiver are in good connection status. - Light Flashing: Transmitter and Receiver are in poor connection status. - Light off: Transmitter and Receiver are not connected. |
| 10 | Data Signal Indicator (Yellow) | - Light Flashing: There is data transmission. - Light off: There is no data transmission. |
| 11 | HDMI OUT port | HDMI signal output port, connected to HDMI display device such as TV or monitor. |
| 12 | AUDIO OUT port | Analog audio output port. |
| 13 | RS-232 port | Connected to PC or control system through 3-pin Phoenix connector for RS-232 serial command pass-through. |
| 14 | IR IN port | Connected to IR wideband receiver cable, receiving the remote control signal and then send it to the transmitting end. |
| 15 | IR OUT port | Connected to IR blaster cable, sending the remote control signal from the transmitting end. |

## Dimensions of Wall Box

38.5mm

70mm

OFF

HDBaseT OUT

RS-232  IR IN IR OUT DC 24V RX TX 5V S

ON

86mm

USB-C IN

USB-C

HDMI IN

HDMI

86mm

AUDIO IN

TO PC

POC

SOURCE

Front panel

Side Panel

Rear Panel

※ Description 1

POC (Power over Cable) Application Example

USB-C IN

CAT5e/6/7

USB-C

HDMI

HDMI IN

AUDIO IN TO PC SOURCE

USB-C IN

USB-C

HDMI

CAT5e/6/7

HDMI IN

AUDIO IN TO PC SOURCE

※ Description 2

Bidirectional Infrared control Application Example

USB-C IN

HDMI Cable

TX

TX 1

USB-C

HDMI

UTP Cable

HDMI Cable

RX

HDMI IN

TX 2

AUDIO IN TO PC SOURCE

TV

TV Remote

DVD Remote

Rx2

Rx1

※ Description 3

Bidirectional RS-232 control Application Example

USB-C IN

USB-C

HDMI

HDMI IN

AUDIO IN TO PC SOURCE

USB-C IN

USB-C

HDMI

HDMI IN

AUDIO IN TO PC SOURCE

# Application Example

## Transmitter

CAT6 Cable

Projector

Speakers

IR Blaster

USB-C IN

USB-C HDMI

IR Receiver

HDMI IN

HDBaseT OUT

OFF

AUDIO IN TO PC  SOURCE

RS-232 IR IN IR OUT DC 24V RX TX 5V S

POC

ON

ON OFF

L R   RX

DC 24V

POC

HDBT IN

HDMI OUT AUDIO OUT RS-232  IR IN IR OUT

Notebook

Power Supply IR Blaster

LINK VIDEO

Receiver

Mouse

1	2

USB 2.0  USB 2.0

SERVICE

Desktop

IR Receiver

Keyboard