# WFH-140

VER 1.2

Thank you for purchasing this product

For optimum performance and safety, please read these instructions carefully before connecting, operating or adjusting this product. Please keep this manual for future reference.

Surge protection device recommended

This product contains sensitive electrical components that may be damaged by electrical spikes, surges, electric shock, lighting strikes, etc. Use of surge protection systems is highly recommended in order to protect and extend the life of your equipment.

Table of Contents

- Introduction.	1

- Features.	1

- Package Contents.	1

- Specifications.	2

- Operation Controls and Functions.	3

  - Encoder Panel.	3

  - Decoder Panel.	4

  - Pairing Instruction.	4

  - IR Pin Definition.	5

- Application Example.	7

# Introduction

This product is based on H.265 standard solution for transmitting one HD source signal to one HD display. It extends distance up to 330ft/100 meters (In an open environment without Wi-Fi interference) between the encoder

and decoder via wireless transmission. It supports one-way IR control. It offers high quality configurable and low-bandwidth H.265 compression video. Input video resolution is up to 1920×1080@60Hz; Output video resolution is up to 1080P@60Hz. This wireless extender is designed special to transmit high definition video & audio within one environment.

# Features

☆ HDMI 1.3, HDCP 1.4 and DVI 1.0 compliant

☆ Transmit one HDMI HD source signal to one HDMI HD display via wireless technology

☆ With one-way IR control

☆ Input video resolution is up to 1920×1080@60Hz YCbCr 4:4:4; Output video resolution is up to 1080P@60Hz YCbCr 4:4:4

☆ HDMI wireless transmission distance up to 330ft/100m. (In an open

environment without Wi-Fi interference.)

☆ Supported HDMI audio formats:

Input: LPCM 2.0CH 32KHz/44.1KHz/48KHz/88.2KHz/96KHz/176.4KHz/ 192KHz

Output: LPCM 2.0CH 48KHz

☆ Adopt H.265 high performance codec technology

☆ End-to-end delay is less than 120Ms

☆ Compact design for easy and flexible installation.

# Package Contents

| Qty | Item |
| --- | --- |
| 1 | HDMI Wireless Extender (Encoder) |
| 1 | HDMI Wireless Extender (Decoder) |
| 1 | IR Blaster cable (1.5 meters) |
| 1 | 20~60KHz IR Receiver cable (1.5 meters) |
| 4 | 5G Wi-Fi Antenna |
| 2 | 5V/2A Multinational Power Supply |
| 1 | User Manual |

# Specifications

| Technical |  |
| --- | --- |
| HDMI Compliance | HDMI 1.3 |
| HDCP Compliance | HDCP 1.4 |
| Video Bandwidth | 4.95Gbps |
| Video Resolution | Input: Up to 1920×1080@60Hz Output: Up to 1080P@60Hz |
| Color Space | RGB, YCbCr 4:4:4 / 4:2:2 |
| Color Depth | 8/10/12-bit (1080P@60Hz) |
| HDMI Audio Formats | Input: LPCM 2.0CH 32KHz/44.1KHz/48KHz/ 88.2KHz/96KHz/176.4KHz/192KHz Output: LPCM 2.0CH 48KHz |
| ESD Protection | Human body model — ±8kV (Air-gap discharge) & ±4kV (Contact discharge) |
| Connection |  |
| Encoder | Inputs: 1 x HDMI IN [Type A, 19-pin female] Outputs: 1 x IR OUT [3.5mm Stereo Mini-jack] 2 x Wi-Fi OUT [Wi-Fi antenna] Control: 1 x SERVICE [Micro USB, Update port] |
| Decoder | Inputs: 1 x IR IN [3.5mm Stereo Mini-jack] 2 x WiFi IN [Wi-Fi antenna] Outputs: 1 x HDMI OUT [Type A, 19-pin female] Control: 1 x SERVICE [Micro USB, Update port] |
| Mechanical |  |
| Housing | Plastic Enclosure |
| Color | Black |
| Dimensions | Encoder / Decoder: 76mm [W] x 98mm [D] x 21mm [H] |
| Weight | Encoder / Decoder: 100g |
| Power Supply | Input: AC100 - 240V 50/60Hz Output: DC 5V/2A |
| Power Consumption | Encoder: 3.5W, Decoder: 2.6W (Max) |
| Operating Temperature | 32 - 104°F / 0 - 40°C |

| Storage Temperature | -4 - 140°F / -20 - 60°C |
| --- | --- |
| Relative Humidity | 20 - 90% RH (no condensation) |
| Resolution / Cable Length | 1080P60Hz - Feet / Meters |
| HDMI IN / OUT | 42ft / 15M |
| The use of “Premium High Speed HDMI” cable is highly recommended. |  |

# Operation Controls and Functions

## Encoder Panel

LINK

HDMI IN

DC 5V

1  2	3

4

8

8

5	6	7

PAIR  SERVICE  IR OUT

| No. | Name | Function Description |
| --- | --- | --- |
| 1 | Power LED | The LED will illuminate red when the encoder is powered on. |
| 2 | LINK LED (Green) | The LINK LED flashing means that the encoder and decoder are paired successfully. (Please refer to “5.3 Pairing Instruction” for details) |
| 3 | HDMI IN | HDMI source input port for connecting the HDMI source device. |
| 4 | DC 5V | Connect the DC 5V/2A power adapter. |
| 5 | PAIR button | - Press and hold the PAIR button, then power on the encoder, the encoder will enter the upgrade mode. - Press and hold the PAIR button for 5 seconds after powering on the encoder, the encoder will restore factory settings, then it needs to pair with the decoder again. |
| 6 | SERVICE port | Firmware update port. |
| 7 | IR OUT | Connect the IR blaster cable. The IR blaster signal is from the IR IN port on the Decoder. |
| 8 | Antenna port | Connect the Wi-Fi antenna. |

8

8

LINK	HDMI OUT

DC 5V

1  2	3

4

5	6	7

PAIR  SERVICE  IR IN

Decoder Panel

| No. | Name | Function Description |
| --- | --- | --- |
| 1 | Power LED | The LED will illuminate red when the decoder is powered on. |
| 2 | LINK LED (Green) | The LINK LED flashing means that the encoder and decoder are transmitting video signals. (Please refer to “5.3 Pairing Instruction” for details) |
| 3 | HDMI OUT | HDMI output port for connecting the HDMI display device. |
| 4 | DC 5V | Connect the DC 5V/2A power adapter. |
| 5 | PAIR button | - Press and hold the PAIR button, then power on the decoder, the decoder will enter the upgrade mode. - Press and hold the PAIR button for 5 seconds after powering on the decoder, the decoder will restore factory settings. |
| 6 | SERVICE port | Firmware update port. |
| 7 | IR IN | Connect the IR receiver cable. The IR signal is sent to the IR OUT port on the encoder. |
| 8 | Antenna port | Connect the Wi-Fi antenna. |

## Pairing Instruction

Follow steps below to pair the encoder and decoder:

HDMI cable

Step 1, Connect the encoder and decoder with an HDMI cable, as shown in the figure below.

Step 2, Power on the decoder, then the power LED and LINK LED of the decoder will be always on.

Step 3, Power on the encoder. After successful pairing, the LINK LED of the decoder will flash. (If the pairing fails, return to Step 2.)

If the LINK LED of encoder is always on, it indicates that the encoder and decoder has not been connected or paired. When there is video signal transmission, the LINK LED of decoder will flash; When there is no video signal transmission, the LINK LED of decoder will be always on.

The specific instructions are shown in the following table.

| Encoder’s LINK LED status (Green) | Decoder’s LINK LED status (Green) | Description |
| --- | --- | --- |
| Always on | Always on | The encoder and decoder are not paired or connected. |
| Flash | Always on | Successful pairing. |
| Flash | Flash | The encoder and decoder are paired successfully, and there is video signal transmission. |

Note: After successful pairing, if the encoder restores factory settings, the encoder and decoder need to be re-paired.

## IR Pin Definition

IR Receiver and Blaster pin’s definition is as below:

IR RECEIVER	IR BLASTER

IR Blaster

IR Receiver

IR Blaster Signal Power

NC

IR Signal

Power Grounding

Connection diagram of using IR cables.

Encoder

PAIR  SERVICE IR OUT

LINK  HDMI IN	DC 5V

DVD

Decoder

DVD remote

TV

PAIR SERVICE  IR IN

LINK  HDMI OUT  DC 5V

Note that when the angle between the IR receiver and the remote control is ± 45 °, the transmission distance is 0-5 meters; when the angle between the IR receiver and the remote control is ± 90 °, the transmission distance is 0-8 meters.

# Application Example

The transmission distance extends up to 100m.

Encoder	Decoder

Power Supply

Power Supply

DVD or Blu-ray Player

TV

The terms HDMI and HDMI High-Definition Multimedia interface, and the HDMI Logo are trademarks or registered trademarks of HDMI Licensing LLC in the United States and other countries.