The first step is to connect the RS232 on the machine to the computer through the Phoenix terminal

Step 2, open the upgrade host computer, configure the correct ports and parameters

Step 3, click "connect". If the version number appears below the tool, it means that the machine has successfully connected to the upper computer and can start the upgrade

Step 4, click "open" and select the correct firmware

Step 5, click "upgrade" to start the upgrade

Step 6: Do not disconnect the power or perform other operations during the upgrade process to prevent upgrade failure

Step 7: After the upgrade is completed, restart the device power supply